Knuckle Boom A-PM

(Grease and Lubrication)

BRANCH:       	TRUCK #:       	MILEAGE:

DATE:       	SERIAL NUMBER:       		VENDOR:

TRUCK HOURS:       	VENDOR ADDRESS:

PURCHASE ORDER:       	DATE PM COMPLETED:       		MAKE & MODEL:

PTO HOUR METER:       		TECHNICIANS NAME:       		INVOICE NUMBER:

* Knuckle Boom A-PM (Grease and Lube) required monthly (or every 50 PTO hours)

* Use a molly-grease that is designed for extreme pressure applications (EP Rated)

* The Grease used also must be rated for temperature protection of at least 300° F.

* Repair Codes:  L – Lubricated with grease     R – Repairs needed     A – Adjusted or Replaced     OK – No further repairs needed

| Knuckle Boom A-PM Instructions: | Greased |
| --- | --- |
| 8. Grease the slide pads in the outrigger legs. Grease only the top and bottom of the legs along the area that the pads contact. |  |
| 9. Fold the knuckle boom back into the transport position, complete this PM form and attach it to the invoice. |  |
| 10. Install PM decal on the hydraulic tank near the filler cap and update it with PTO hours, date, and the PM type = A - PM |  |
| 11. Check oil level in hydraulic tank: Boom and outriggers must be in transport position. Fill as needed with manufacturers recommended hydraulic oil only. |  |
| NOTE: Report any obvious hydraulic oil leaks, badly chaffed hoses, or other defects observed while applying grease. |  |

| Knuckle Boom A- PM Instructions: | Greased |
| --- | --- |
| 1. Operate and open the boom above the body fully extending each section. ** Place the boom in a safe position |  |
| 2. Remove excess grease from all of the extensions and pressure wash the boom if necessary. * Remove excess grease during greasing process as needed. |  |
| 3. Grease the fittings in the three piece bridge. |  |
| 4. Grease the Slewing Housing Bushing that supports the entire mast. * Rotate the mast while greasing. * There are Two (2) fittings in the bushing. |  |
| 5. Grease all of the boom pivot pins and lift cylinder anchor pins. Grease the jib pivot pins and bushings. |  |
| 6. Grease the slide pads (Wear pads) in each extension & grease the entire length of the wear surface that the pad contacts. |  |
| 7. Grease the fittings in the load hook and grease the fittings in the control levers. * Inspect the load hook safety latch. * Inspect safety pins in hook anchor pins. |  |

Repairs Needed:

Technicians Signature:  	Print Name:

Copyright © 2009 National Safety Compliance, Inc. 1970 Swarthmore Ave. Lakewood, NJ 08701 Rev. 2007

www.nscsafe.com