DAILY/WEEKLY TRUCK MOUNTED FORKLIFT INSPECTION REPORT

Operator’s Name       	Branch       	Unit #       	Ending Hours Meter Reading

Week beginning       ,	20  	Sunday	Monday	Tuesday	Wednesday	Thursday		Friday	Saturday

| ACTION/DESCRIPTION | OK | Needs Attn. | OK | Needs Attn. | OK | Needs Attn. | OK | Needs Attn. | OK | Needs Attn. | OK | Needs Attn. | OK | Needs Attn. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Check for broken, missing or damaged parts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Check the forks, mast, and lift chains. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Check all hydraulic hoses and connections. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Check the wheels for any damage, missing, or loose lug nuts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Check the tires for cuts, bulges, tread depth, and proper inflation pressure. ( see tire inflation chart). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.Check the engine oil level. Add oil if required. (See section “engine oil”.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.If operating in a dusty or sandy environment; a) Clean engine air filter element and replace it if necessary. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b) Clean all dust etc. from between radiator fins. Where necessary, remove radiator for proper cleaning. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. Lift the hood and check the engine cooling system. Fill to the proper level if necessary. The proper coolant level is just below the radiator cap when the engine is not hot. Escaping steam could cause severe burning. (See section “Engine coolant”.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9. Check the fuel level and top off if necessary. Ground the nozzle against the filler neck to prevent sparks. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. Check hydraulic oil level sight-glass. Oil level must be between the red line (minimum) and the black line (maximum). Top off if necessary. (See section “Hydraulic Fluid”.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks:

____________________________

Signature

Copyright © 2009 National Safety Compliance, Inc. 1970 Swarthmore Ave. Lakewood, NJ 08701 Rev. 2007

www.nscsafe.com